AXI4-Lite custom PL peripheral controlled by ARM

1.Document Information

| Item | Description |
| --- | --- |
| Design Name | axi_ledseg_irq |
| Protocol | AMBA AXI4-Lite |
| Design Type | Memory-mapped peripheral |
| Clock Domain | Single (ACLK) |
| Reset Type | Active-low asynchronous reset |

2. Design Overview

The axi_ledseg_irq module is a memory-mapped AXI4-Lite slave peripheral that provides:

- Control of an 8-bit LED output

- Control of an 8-bit seven-segment display

- An edge-triggered interrupt (IRQ) based on LED state

- Software-accessible status and control registers

The design supports simultaneous read and write transactions using independent read and write finite state machines (FSMs), compliant with AXI4-Lite protocol rules.

3. Functional Features

3.1 Supported Features

- AXI4-Lite compliant read/write interface

- Independent read and write channels

- One outstanding read and one outstanding write supported

- Byte-level write support via WSTRB

- Write-1-to-clear (W1C) interrupt register

- Edge-detected interrupt generation

- Fully synchronous register access

3.2 Unsupported Features

- Burst transfers

- Multiple outstanding reads or writes

- Out-of-order responses

- Unaligned accesses

- AXI IDs

4. Block Design

5.Interface Description

5.1 Clock and Reset

| Signal | Direction | Description |
| --- | --- | --- |
| ACLK | Input | AXI clock |
| ARESETn | Input | Active-low asynchronous reset |

5.2 AXI4-Lite Write Interface

| Signal | Direction | Description |
| --- | --- | --- |
| S_AWADDR[31:0] | Input | Write address |
| S_AWVALID | Input | Write address valid |
| S_AWREADY | Output | Write address ready |
| S_WDATA[31:0] | Input | Write data |
| S_WSTRB[3:0] | Input | Byte write strobes |
| S_WVALID | Input | Write data valid |
| S_WREADY | Output | Write data ready |
| S_BRESP[1:0] | Output | Write response (OKAY only) |
| S_BVALID | Output | Write response valid |
| S_BREADY | Input | Write response ready |

AXI Write Channel – Handshake Signals

Write Address Handshake

| Signal | Direction | Used by TB / Master |
| --- | --- | --- |
| S_AWREADY | Output | Indicates slave is ready to accept write address |

Handshake condition:

S_AWVALID && S_AWREADY

TB waits for S_AWREADY == 1 before deasserting S_AWVALID
 Confirms write address acceptance

Write Data Handshake

| Signal | Direction | Used by TB / Master |
| --- | --- | --- |
| S_WREADY | Output | Indicates slave is ready to accept write data |

Handshake condition:

S_WVALID && S_WREADY

TB waits for S_WREADY == 1
 Confirms write data transfer

Write Response Handshake

| Signal | Direction | Used by TB / Master for |
| --- | --- | --- |
| S_BVALID | Output | Indicates write response is available |
| S_BRESP[1:0] | Output | Indicates write response status |

Handshake condition:

S_BVALID && S_BREADY

TB asserts S_BREADY and waits for S_BVALID
 Confirms write transaction completion
 S_BRESP checked for OKAY (2'b00)

5.3 AXI4-Lite Read Interface

| Signal | Direction | Description |
| --- | --- | --- |
| S_ARADDR[31:0] | Input | Read address |
| S_ARVALID | Input | Read address valid |
| S_ARREADY | Output | Read address ready |
| S_RDATA[31:0] | Output | Read data |
| S_RRESP[1:0] | Output | Read response (OKAY only) |
| S_RVALID | Output | Read data valid |
| S_RREADY | Input | Read data ready |

AXI Read Channel – Handshake Signals

Read Address Handshake

| Signal | Direction | Used by TB / Master |
| --- | --- | --- |
| S_ARREADY | Output | Indicates slave is ready to accept read address |

Handshake condition:

S_ARVALID && S_ARREADY

TB waits for S_ARREADY == 1
 Confirms read address acceptance

Read Data Handshake

| Signal | Direction | Used by TB / Master |
| --- | --- | --- |
| S_RVALID | Output | Indicates read data is valid |
| S_RDATA[31:0] | Output | Read data returned by slave |
| S_RRESP[1:0] | Output | Read response status |

Handshake condition:

S_RVALID && S_RREADY

TB waits for S_RVALID == 1
 Samples S_RDATA
 Checks S_RRESP == OKAY

5.4 External Outputs

| Signal | Direction | Description |
| --- | --- | --- |
| LED_OUT[7:0] | Output | LED drive output |
| SEVENSEG_OUT[7:0] | Output | Seven-segment drive output |
| IRQ_OUT | Output | Interrupt request output |

6. Register Map

All registers are 32-bit, word-aligned, and byte-addressed.

| Address Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x00 | LED Register | R/W | Controls LED output |
| 0x04 | Seven-Segment Register | R/W | Controls seven-segment output |
| 0x08 | IRQ Status Register | R/W1C | Interrupt status |

7. Register Descriptions

7.1 LED Register (Offset 0x00)

| Bit | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| [7:0] | LED_DATA | R/W | 0x00 | Drives LED_OUT |
| [31:8] | Reserved | — | 0 | Read as zero |

- Writing updates LED_OUT

- Supports byte writes via WSTRB

7.2 Seven-Segment Register (Offset 0x04)

| Bit | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| [7:0] | SEG_DATA | R/W | 0x00 | Drives SEVENSEG_OUT |
| [31:8] | Reserved | — | 0 | Read as zero |

- Raw segment control (no digit decoding)

- Supports byte writes via WSTRB

7.3 IRQ Status Register (Offset 0x08)

| Bit | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| [0] | IRQ_STATUS | R/W1C | 0 | Interrupt status |
| [31:1] | Reserved | — | 0 | Read as zero |

- Bit[0] = 1 → IRQ asserted

- Write 1 to write bit[0] clears IRQ

- Write 0 has no effect

8. Interrupt Behaviour

8.1 IRQ Generation

- IRQ is generated when:

  - LED_OUT transitions from any value ≠ 0xFF to 0xFF

- Edge detection is performed internally using a delayed LED value

8.2 IRQ Clearing

- IRQ is cleared by writing 1 to IRQ_STATUS[0]

- Writing 0 does not clear IRQ

8.3 IRQ Output

- IRQ_OUT directly reflects IRQ_STATUS[0]

9. AXI Protocol Compliance

9.1 Read Channel

- One outstanding read supported

- Address and data channels are independent

- Read data is returned with RRESP = OKAY

9.2 Write Channel

- One outstanding write supported

- Address and data phases are sequential

- Write response always BRESP = OKAY

9.3 Simultaneous Read & Write

- Read and write FSMs are independent

- Read and write transactions may occur concurrently

- No ordering guarantee between read and write completion

10. Reset Behaviour

- Active-low asynchronous reset (ARESETn)

- On reset:

  - All registers cleared to zero

  - Outputs deasserted

  - FSMs return to idle state